The goal of the assignment is use GPIO, delays, and Interrupts:

1. Design a delay subroutine to generate a delay of 0.15 sec.

2. Connect a switch to PORTC.1 to poll for an event to turn on the led at PORTB.5 for 1.5 sec after the event.

3. Continue with task 2, connect a switch to INT0 (PD2 pin) (active high - turn on the pull up transistor) and using an interrupt mechanism turn on the led at PORTB.5 for 3 sec after the event.

4. Submit codes and demos for three tasks, one for (2), another for (3), and another for (2 & 3) working together. Verify the delays using simulation and logic analyzer.

Components Used/Connected

Task 1

1. ; AssemblerApp_DA2_1.asm

2. ;

3. ; Created: 3/9/2025 7:21:43 PM

4. ; Author : enriq

5. ;

6.

7. .include "M328Pdef.inc"

8. .equ Value = 0xFB2C ;Timing value for 0.15 sec delay

9.

10. Main:

11.     rcall Delay         ; Call the 0.15 second delay

12. 	nop

13. Done:

14.     rjmp Done           ; Loop forever

15.

16. Delay:                  ; ~0.15 second delay using Timer1

17.     ldi R30, high(Value)

18.     sts TCNT1H, R30

19.     ldi R30, low(Value)

20.     sts TCNT1L, R30     ; Load TCNT1 with starting value

21.

22.     ldi R31, 0x00

23.     sts TCCR1A, R31     ; Normal mode

24.

25.     ldi R31, 0x05       ; Prescaler 1024

26.     sts TCCR1B, R31     ; Start Timer1

27.

28. Wait:

29.     sbis TIFR1, TOV1    ; Wait for overflow flag

30.     rjmp Wait

31.

32.     sbi TIFR1, TOV1     ; Clear overflow flag

33.     ldi R30, 0x00

34.     sts TCCR1B, R30     ; Stop Timer1

35.

36.     ret

37.

1. /*

2.  * GccApp_DA2_1.c

3.  *

4.  * Created: 3/9/2025 7:55:28 PM

5.  * Author : enriq

6.  */

7.

8. #define F_CPU 8000000UL

9.

10. #include <avr/io.h>

11. #include <util/delay.h>

12.

13. int main(void)

14. {

15. 	_delay_ms(150);

16.

17. 	return 0;

18. }

19.

Task 2

1. /*

2.  * DA2_2_C.c

3.  *

4.  * Created: 3/21/2025 8:58:26 PM

5.  * Author : enriq

6.  */

7.

8. #include <avr/io.h>

9.

10.

11. #define F_CPU 8000000UL

12.

13. #include <avr/io.h>

14. #include <util/delay.h>

15.

16. int main(void)

17. {

18. 	// Set PORTB5 as output (for LED)

19. 	DDRB |= (1 << PORTB5);

20.

21. 	// Set PORTC1 as input (for switch)

22. 	DDRC &= ~(1 << PORTC1);

23.

24. 	PORTC |= (1 << PORTC1);

25.

26. 	while (1)

27. 	{

28. 		// check if pressed (low)

29. 		if (!(PINC & (1 << PINC1)))

30. 		{

31. 			// Turn LED on

32. 			PORTB |= (1 << PORTB5);

33.

34. 			// Delay for 1.5 seconds

35. 			_delay_ms(1500);

36.

37. 			// Turn LED off

38. 			PORTB &= ~(1 << PORTB5);

39. 		}

40. 	}

41.

42. 	return 0;

43. }

44.

1. ;

2. ; DA2_2_asm.asm

3. ;

4. ; Created: 3/21/2025 11:29:28 PM

5. ; Author : enriq

6. ;

7.

8. .include "M328Pdef.inc"

9. .equ Value = 0xFB2C       ; Timer preload for ~0.15 sec

10. .equ DelayCount = 10      ; 10 * 0.15 sec = 1.5 sec

11.

12.

13. Main:

14.     ; Set PORTB5 as output

15.     sbi DDRB, DDB5

16.

17.     ; Set PORTC1 as input

18.     cbi DDRC, DDC1

19.

20.     ; Enable internal pull-up on PORTC1

21.     sbi PORTC, PORTC1

22.

23. Loop:

24.     ; Check if button on PORTC1 is pressed (low)

25.     sbic PINC, PINC1       ; Skip next if PINC1 is pressed

26.     rjmp Loop              ; Not pressed, loop again

27.

28.     ; Turn on LED on PORTB5

29.     sbi PORTB, PORTB5

30.

31.     ; Delay for 1.5 seconds using 10x 0.15 sec Delay

32.     ldi R20, DelayCount

33. DelayLoop:

34.     rcall Delay

35.     dec R20

36.     brne DelayLoop

37.

38.     ; Turn off LED on PORTB5

39.     cbi PORTB, PORTB5

40.

41.     rjmp Loop              ; Wait for next press

42.

43. ; 0.15 Second Delay

44.

45. Delay:

46.     ldi R30, high(Value)

47.     sts TCNT1H, R30

48.     ldi R30, low(Value)

49.     sts TCNT1L, R30         ; Load TCNT1 with starting value

50.

51.     ldi R31, 0x00

52.     sts TCCR1A, R31         ; Normal mode

53.

54.     ldi R31, 0x05           ; Prescaler 1024

55.     sts TCCR1B, R31         ; Start Timer1

56.

57. Wait:

58.     sbis TIFR1, TOV1        ; Wait for overflow flag

59.     rjmp Wait

60.

61.     sbi TIFR1, TOV1         ; Clear overflow flag

62.     ldi R30, 0x00

63.     sts TCCR1B, R30         ; Stop Timer1

64.

65.     ret

66.

Task 3

1. /*

2.  * DA2_3_C.c

3.  *

4.  * Created: 3/21/2025 11:46:25 PM

5.  * Author : enriq

6.  */

7.

8. #define F_CPU 8000000UL

9.

10. #include <avr/io.h>

11. #include <avr/interrupt.h>

12. #include <util/delay.h>

13.

14.

15. void setup()

16. {

17. 	DDRB |= (1 << DDB5);      // LED as output

18.

19. 	DDRD &= ~(1 << DDD2);     // PD2 as input

20. 	PORTD |= (1 << PORTD2);   // Enable internal pull-up

21.

22. 	// INT0 triggers on falling edge

23. 	EICRA |= (1 << ISC01);

24. 	EICRA &= ~(1 << ISC00);

25.

26. 	EIMSK |= (1 << INT0);     // Enable INT0

27.

28. 	sei();                    // Global interrupt enable

29. }

30.

31. int main(void)

32. {

33. 	setup();

34.

35. 	while (1)

36. 	{

37.

38. 	}

39. }

40.

41. ISR(INT0_vect)

42. {

43. 	PORTB |= (1 << PORTB5);   // LED ON

44. 	_delay_ms(3000);          // Wait 3 seconds

45. 	PORTB &= ~(1 << PORTB5);  // LED OFF

46. }

47.

1. ;

2. ; DA2_3_asm.asm

3. ;

4. ; Created: 3/22/2025 6:27:19 PM

5. ; Author : enriq

6. ;

7.

8. .include "M328Pdef.inc"

9. .equ Value       = 0xFB2C      ; Timer preload for ~0.15 sec

10. .equ DelayCount  = 20          ; 20 x 0.15 s = 3 s total

11.

12. .cseg

13. .org 0x0000

14.     rjmp Setup

15.

16. .org 0x0002                  ; INT0 vector

17.     rjmp INT0_ISR

18.

19.

20. Setup:

21.     ldi r16, high(RAMEND)

22.     out SPH, r16

23.     ldi r16, low(RAMEND)

24.     out SPL, r16

25.

26.     sbi DDRB, DDB5            ; Set PB5 as output

27.

28.     cbi DDRD, DDD2            ; PD2 input

29.     sbi PORTD, PORTD2         ; Enable pull-up on PD2

30.

31.

32.     ; set INT0

33.     ldi r16, (1 << ISC01)

34.     sts EICRA, r16

35.

36.     ; Enable INT0 interrupt

37.     ldi r16, (1 << INT0)

38.     sts EIMSK, r16

39.

40.     sei	; Global interrupt enable

41.

42. Main:

43.     rjmp Main

44.

45. INT0_ISR:

46.     sbi PORTB, PORTB5	; Turn on LED at PB5

47.     ldi r20, DelayCount

48.

49. DelayLoop:

50.     rcall Delay

51.     dec r20

52.     brne DelayLoop

53.

54.     ; Turn off LED at PB5

55.     cbi PORTB, PORTB5

56.

57.     reti

58.

59.

60. Delay:                  ; ~0.15 second delay using Timer1

61.     ldi R30, high(Value)

62.     sts TCNT1H, R30

63.     ldi R30, low(Value)

64.     sts TCNT1L, R30     ; Load TCNT1 with starting value

65.

66.     ldi R31, 0x00

67.     sts TCCR1A, R31     ; Normal mode

68.

69.     ldi R31, 0x05       ; Prescaler 1024

70.     sts TCCR1B, R31     ; Start Timer1

71.

72. Wait:

73.     sbis TIFR1, TOV1    ; Wait for overflow flag

74.     rjmp Wait

75.

76.     sbi TIFR1, TOV1     ; Clear overflow flag

77.     ldi R30, 0x00

78.     sts TCCR1B, R30     ; Stop Timer1

79.

80.     ret

81.